# CEN-NVS100 Configuration Quickstart

# Installation

- Device requires PoE for power.

- Audio Output is for door speaker.

- Audio Input is for door microphone.

- 12V power is not supported (neither barrel plug nor 6 pin P/C wires).

- Cuz 4.001.1012 is required

# Configuration

## Find the device on the network

- Open the Toolbox Device Discovery Tool

- Find the device labeled “CEN-NVS100” on the left pane and select it.  The device webpage should appear.  Note that the device may take some time from power up to detection time.  Simply press Refresh to redetect the device.

Figure 1 NVS100 Home Page

## Select “Configuration”

Figure 2 System Page

- You can change the Host Name to something unique for your environment – for example “Front Camera.”  This name is not used as a network name but does display when searching the Windows Network.  Press Save at the bottom of the screen.

- From the System page you can also configure the Digital I/O or hide the controls altogether if they are not being used.

## Select “Security”

Figure 3 Security Page – Root Password

- Enter a root password – if you do not set a root password then anyone can access any setting on the camera.  Press save – note that the browser will ask for a username and password.  The username is root, the password is whatever you set it to.

- It is recommended that you add an additional user of type “viewer.”

Figure 4 Security Page - Add a Viewer Level User

## Select “Network”

- The device is set to use DHCP by default for the ease of installation.  It is highly recommended that you choose a static address.  Make sure not to use an address in the range of the dynamically assigned DHCP addresses.  Press the save button directly beneath this section.

- Also make sure the HTTP port that is set on the NVS100 is the same as the “Vid_Server_Port_Num” parameter on the NVS100 Module (See figure 6).

Figure 5 Set Network Settings

Figure 6 Set Network Settings – HTTP Port Setting

## Select “Camera Control”

- If you have a PTZ camera you can set it up here.

Figure 7 Camera Control

## To Setup A Preset Position

Click on “Preset Position” on the “Camera Control” page to get to the “Preset Position” page.

### To Add a preset Position

- Go to the “Preset Position” page.

- Use the Camera Controls to move the camera to the desired preset position.

- Enter the Preset Name into the “Preset Position Name” textfield

- Click Add to add the Preset position

### To Remove A Preset Position

- Go to the “Preset Position” page

- Click on the drop down menu from the “Preset Position”.

- Select the Preset position you wish to remove

- Click on the “Delete” button.

Figure 8 Camera Control – Preset Setup

## To Setup A Custom Command

Click on “Custom Command” on the “Camera Control” page to get to the “Custom Command” page.

### To Modify a Custom Command

- Enter the Custom Command into the “Command” Text Field. (Command string format can be found using the manual that came with your camera and differs from camera to camera.)

- Enter the Command name into the “Button Name” Text Field.

- Click “Save” to save your changes.