# CEN-NVS100 Programming

Control system programming is required to use PTZ and to receive notification of online status, video loss, and motion detection.  If you decide you do not need any of these features you may skip to here.

## SIMPL Windows Configuration View

- Select the CEN-NVS100 from the Ethernet Video Modules section of the device library and add it to the IP-ID of your choice.

Figure 1 NVS100 dropped onto IPID 0F

- Right click on the NVS100 device and Select Configure.

- From the window that pops up, click on the "IP Net Address" tab and enter the device’s IP Address into the Default Address field.

## SIMPL Windows Programming View

- Look for the CEN-NVS100 module in the Logic section of your program.

- Go to the bottom of the module and make sure to fill in your settings for

  - Root_Password

  - Viewer_Name (the viewer level user you created earlier)

  - Viewer_Password

  - IP_ID – You must manually set this to the IP ID you dropped the NVS100 into (in our example 0F).

  - Vid_Server_Port_Num (The port number you configured the NVS100 to use.)

  - Cntrl_Sys_Port_Num (The port number the Control System will use to listen to response from the video server.)

Figure 2 Parameter Settings in the NVS100 Module

- Hook up the PTZ and other joins as desired (press F1 on the module for help)

- In particular connect the HTTP Command, IP_Adress and Port number outputs to the appropriate inputs on the touchpanel.  In the example below we connect HTTP_Command and IP_Address to serial joins 1 and 2 and Port_Number to Analog Join 1.

Figure 3 NVS100 Module Outputs

Figure 4 Link to TPMC Serial Joins

Figure 5 Link to TPMC Analog Joins

## VTPRO

Launch the MJPEG Viewer properties, which can be found under Embedded Applications:

Set the IP Address and HTTP Command serial join to match the serial joins shown in figure 9 (2 and 1 in this example).

Set the Port Number analog join to match the port number analog join used in figure 10 (1 in this example).

### Alternate Instructions with a Touchpanel Only

If you are not using the module you can simply type in the IP Address and Port number in the appropriate fields.

The HTTP Command string should be set to

/video.mjpg?user=<myusername>&pw=<myuserpw>