AMS Theater and Four Audio Zones - Simplified v2.0.0 Release Notes.txt

AMS Theater and Four Audio Zone – Simplified v2.0.0.0.smw  Release Notes

This program implements a Theater and 4 rooms of Adagio AMS distributed audio, which includes control of an ATC-AMFMXM AM/FM/XM tuner card and an ATC-AMFMSR AM/FM/Sirius tuner card. Control for an AAS-1, CEN-IDOC, Dish Network HD-DVR satellite receiver, HDDVD player (Microsoft XBOX360), Denon Multi-Disc CD Changer and a Sony television.

A C2N-DBF12 has been included for control of each distributed audio room. A TPMC-8X has been included for control of the Theater, as well as the 4 rooms of distributed audio. An ML-600 has been included for convenient control of the Theater.

This program contains a default IP Table. When loading the system, please be sure to load the default IP Table when prompted by Toolbox. Also be sure to configure the IP Tables on the AAS-1, CEN-IDOC and TPMC-8X to point to the AMS control processor. The correct IP-IDs are shown below.

Configuration

This program is designed to work with the following hardware:

# Adagio AMS Control System

Card Slot 1 – ATC-AMFMXM AM/FM/XM Tuner Card

Card Slot 2 – ATC-AMFMSR AM/FM/Sirius Tuner Card

Card Slot 3 – Empty

IR Port A - Unused

IR Port B – Dish Network HD DVR Satellite Receiver

IR Port C – HDDVD Player (Microsoft XBOX360)

IR Port D - Denon Multi-Disc CD Changer

IR Port E – Sony Television (Universal Discrete Codes Only)

IR Port F – Unused

IR Port G – Unused

IR Port H – Unused

# Ethernet Devices

IP-ID 03 – CEN-IDOC

IP-ID 04 – AAS

IP-ID 0A – TPMC-8X

Page 1

# Cresnet Devices

# ID 03 – C2N-DBF12 – Configured for operation of Room 1.

ID 04 – C2N-DBF12 – Configured for operation of Room 2.

ID 05 – C2N-DBF12 – Configured for operation of Room 3.

ID 06 – C2N-DBF12 – Configured for operation of Room 4.

ID 24 – CNRFGWA-418 – Configured for reception of RF signals from ML-600.

1. RF ID 00- ML-600

The Sources are defined as follows:

This information can be found on the AMS Standard Functionality v1_0_2(cm) module at the Theater Source “” Name$ parameter field.

Source 1 = AM/FM

Source 2 = XM

Source 3 = {{AM/FM (Not used in this program)}}

Source 4 = Sirius

Source 5 = CEN-IDOC

Source 6 = HD DVD

Source 7 = AAS-1

Source 8 = HD DVR

Source 9 = CD Changer

# The physical input connections are as follows

Source 1 = AM/FM (Card Slot 1)

Source 2 = XM (Card Slot 1)

Source 3 = {{AM/FM (Card Slot 2) Not used in this program}}

Source 4 = Sirius (Card Slot 2)

Source 5 = CEN-IDOC – No Video Input and Stereo Analog Input 5

Source 6 = HD DVD – HDMI 2 (Audio and Video)

Source 7 = AAS-1 – Composite Video 1 and Stereo Analog Input 7

Source 8 = HD DVR – HDMI 1 (Audio and Video)

Source 9 = CD Changer – No Video Input and Stereo Analog Input 6

# Software Versions

Simpl Windows  – 2.08.44 Device Library 508/Symbol Library 508  or Later

VTPro-e  – 3.7.2.1 or Later

Crestron Database  – 19.0.4 or Later

# Firmware Versions

AMS – 3.158.0011 or Later

TPMC-8X – 1.00.00.41 or Later

Page 2

Important Information

To modify this program for use with your own equipment is a simple process. All third party controlled devices are based on IR controlled equipment. This allows for changing of IR drivers to gain control of a different manufacturers device. To change the input of a device in the system you will need to open the AMS Standard Functionality v1_0_2(cm) module in the Detail View of Simpl Windows. Scroll down to the parameter fields of the AMS Standard Functionality v1_0_2(cm) module and locate the following parameter fields:

Theater Source “” Name$ - This is where you will name the source. (Source 1 through 9 are defined within this demo program)

Theater Source “” Audio Input – This is where you will define the input type for the source you are adding/changing. Please be aware that the tuner cards in Slot 1 and Slot 2 are occupying Stereo Analog Inputs 1,2, 3 and 4. These inputs are pre-defined based on the Tuner Card Slots.

Theater Source “” Video Input – This is where you will define the video input type for the source you are adding/changing.

For example, if you were changing the HD DVR from an HDMI input to a Component Video Input with a Digital Coaxial Audio input you would define Theater Source 5 Audio Input to a value of “Digital Audio 1 [12d]” and Theater Source Video 5 Input to a value of “Component 1 [1d]” by using the available drop down menus.

To add a source to the system you will need to do the following:

- Assign the Audio and Video inputs on the AMS Standard Functionality v1_0_2(cm) module. You will start with Source 10.

- Expand S-2.2.1 and S-2.2.2 and assign the new source selection button signal name. You will also need to assign a parameter value. This value is the same value as the Theater Source analog value for the source switching (S-4.1.1 and S-4.1.2). These modules control the navigation of pages within the TPMC-8X.

- Expand S-2.2.6 to allow for the new source and assign the source selection button press signal name to the new input.

- Expand S-4.1.1 and S-4.1.2 and assign the new source selection button signal name. You will also need to assign a parameter value. This value is the value of the new source selection. For source 10 the value will be 10d. These modules control source selection and real-time feedback to/from the AMS.

- Open S-5.5 and expand the inputs of this Serial I/O to allow for a new source. Assign the same signal name you have used in the above symbols to drive this serial parameter field. You will also need to assign the source name in the new parameter field. This Serial I/O drives the page names on the TPMC-8X.

Page 3

- Program your source just as you would any other source in a Simpl Windows environment. You will need to include a new subsystem folder under S-3: Sources folder. Within this folder you will include logic for controlling your new device. You will need to assign the button presses/feedbacks to the TPMC-8X.

- Within VTPro-e you will need to create the source page, assign a join number to that page and design the controls for your source. You will also need to assign the join numbers you have assigned in Simpl Windows to the buttons you have created in VTPro-e.

- You will also need to locate the Home Theater Source Bar subpage within the Theater folder. You will add your new source selection button to this page. You do not need to assign a page flip to this button as the logic in Simpl Windows drives the page.

- Within Simpl Windows you will need to assign your new source functions to the ML-600.

- Compile and load the AMS processor from within Simpl Windows.

- Compile and load the TPMC-8X from within VTPro-e.

- Update the pages for the ML-600 within VTPro-e.

- Compile and upload the ML-600.

To adjust room names and sources for the rooms you will need to do the following:

- Open the AMS Standard Functionality v1_0_2(cm) module and locate the Room “” Name$ parameter fields. Edit the names as needed.

- Open symbols S-5.1, S-5.2, S-5.3 and S-5.4. Edit the Room Names in the parameter fields as needed.

- Open the AMS Standard Functionality v1_0_2(cm) module and locate the Source “” Name$ parameter field. Edit the Source Names as needed.

- Open modules S-4.2.1.3, S-4.2.2.3, S-4.2.3.3 and S-4.2.4.3. Edit the Source Names as needed.

You should now have a new source added to your system and have control of the new source.

Page 4